GENESIS

Learning Report on

Embedded C – Hardware + Programming + Testing

(MBSE)

./

| Ver. Rel. No. | Release Date | Prepared. By | Reviewed By | To be Approved | Remarks/Revision Details |
| --- | --- | --- | --- | --- | --- |
| 1. | 27-2-2021 | Reeshav Rout |  |  |  |

Details

TABLE OF CONTENTS

TABLE OF CONTENTS	3

ACTIVITY 1	5

ACTIVITY 2	6

ACTIVITY 3	9

ACTIVITY 4	10

ACTIVITY 5 : IMPLEMENTATION OF GPIO, EXTI, ADC, SPI AND UART USING STM IDE…………………

5.1 LED Toggle using Push Pull Configuration on STM32 Board.	11

5.2 LED Toggle using Open Drain configuration and enabling Internal Pull Up on STM32 Board.	13

5.3 LED Toggle using open drain configuration and external pull up on STM32 Board.	14

5.4 Toggle External LED using push pull configuration on STM32 Board.	15

5.4 Enable internal button and toggle the LED using STM32 Board.	16

5.6 Use external switch to toggle the LED on STM32 Board.	18

5.7 HAL INTERRUPT	20

5.8 HAL ADC	23

5.9 HAL SPI	25

5.10 HAL UART	26

LIST OF FIGURES

Figure 1 : MakeFile command	Error! Bookmark not defined.

Figure 2 : Makefile code	Error! Bookmark not defined.

Figure 3: StartUp file	Error! Bookmark not defined.

Figure 4: Linker Script	Error! Bookmark not defined.

Figure 5: LED toggle code	11

Figure 6: LED toggle output	12

Figure 7: open drain internal pull up code	13

Figure 8:open drain internal pull up output	13

Figure 9:open drain external pull up code	14

Figure 10: open drain external pull up output	14

Figure 11: External LED Push pull code	15

Figure 12: External LED Push Pull output	15

Figure 13: internal button led toggle code	16

Figure 14:internal button led toggle code	17

Figure 15:internal button led toggle output	17

Figure 16:external switch led toggle	18

Figure 17:external switch led toggle code	19

Figure 18: external switch led toggle output	19

Figure 19: HAL interrupt	20

Figure 20:  HAL interrupt	20

Figure 21: HAL interrupt	21

Figure 22:  HAL interrupt	21

Figure 23: HAL interrupt	22

Figure 24: HAL interrupt	22

Figure 25: HAL ADC	23

Figure 26: HAL ADC	23

Figure 27: HAL ADC	24

Figure 28: HAL SPI	25

Figure 29: Output of SPI	26

Figure 30: Output of UART	26

# ACTIVITY 1

CREATE A MAKEFILE

### MAKEFILE

Make is UNIX utility that is designed to start execution of a make file. A makefile is a special file, containing shell commands that user creates and name makefile (or Makefile depending upon the system). While in the directory containing this makefile, user needs to type make and the commands in the makefile will be executed.

Make keeps track of the last time files (normally object files) were updated and only updates those files which are required (ones containing changes) to keep the source file up-to-date. If you have a large program with many source and/or header files, when you change a file on which others depend, you must recompile all the dependent files. Without a makefile, this is an extremely time-consuming task.

As a makefile is a list of shell commands, it must be written for the shell which will process the makefile. A makefile that works well in one shell may not execute properly in another shell.

The makefile contains a list of rules. These rules tell the system what commands you want to be executed. Most times, these rules are commands to compile (or recompile) a series of files. The rules, which must begin in column 1, are in two parts. The first line is called a dependency line and the subsequent line(s) are called actions or commands. The action line(s) must be indented with a tab.

### 1.2 CREATING A MAKEFILE:

Procedure done:

- Dividing the program into multi files (.c and .h)

- Object file designed by the command : -o

- The command : arm-none-eabi-gcc main.c –o main.o  -

It takes the input file main.c and creates the relocation object file (main.o).

- arm-none-eabi-gcc –c –mcpu=cortex-m4-mthumb main.c –o main.o –

This removes the error in assembler stage.

The makefile command:

Figure 1: Makefile command

Figure 2: Code snippet of Makefile

# ACTIVITY 2

CREATE A STARTUP FILE

2.1 START UP FILE:

A start up file is a piece of code written in assembly or C language that executes before the main () function of our embedded application. It performs various initialization steps by setting up the hardware of the microcontroller so that the user application can run. Therefore, a start-up file always runs before the main () code of our embedded application.

### 2.2 CREATING A START UP FILE:

Following are the main functions of a start-up file:

- Disable all interrupts.

- Copying initialized global, global static, and local static variable data from flash to .data section RAM memory of a microcontroller

- Copying uninitialized global, global static, and local static variable data from flash to .bss section of RAM memory and initialize .bss section of RAM to zero.

- Allocate space for the stack and initialize the stack pointer

- It also contains an array of function pointers (interrupt vector table) that point to various interrupt vector routines such as interrupts and exceptions. The start-up file also contains definitions of these interrupt or exception routines such as reset handler, NMI handler, bus fault handler, etc.

- Enable interrupts

- Calls the main function

Figure 3: Code snippet of start up file

# ACTIVITY 3

CREATE A LINKER SCRIPT

3.1 LINKER SCRIPT:

The Linker Script is a text file made up of a series of Linker directives which tell the Linker where the available memory is and how it should be used. Thus, they reflect exactly the memory resources and memory map of the target microcontroller.

3.2 CREATING A LINKER SCRIPT :

Figure 4: Code snippet of Linker script

# ACTIVITY 4

DEBUGGING TECHNIQUES

Debugging is the process of detecting and removing of existing and potential errors (also called as ‘bugs’) in a software code that can cause it to behave unexpectedly or crash. To prevent incorrect operation of a software or system, debugging is used to find and resolve bugs or defects. When various subsystems or modules are tightly coupled, debugging becomes harder as any change in one module may cause more bugs to appear in another.

Executable Download:

Connect the target to the PC by debug adapter, instituting programming. The host protocol to the native target protocol.

Programming Adapters:

It is used to get access to debug interface of the target with native protocol. It helps to download and debug code.

Eg: SEGGER - JLINK EDU

Open OCD:

Procedure:

1. Open OCD connects to debug adopter over its driver.

2. Open OCd sends USB packet as a target interface.

3. You can use JTAG pins or SWD. SWD has two pins. i.e., Clock and IO pins. IO is a data line> It is controlled by debug adopter and it bidirectional.

Various bus interface can be accessed such as ABP. There are various access points in the processors to talk to memory and core.

ACTIVITY 5

TOGGLE LED

# 5.1 LED Toggle using Push Pull Configuration on STM32 Board.

Figure 1: LED toggle code

Figure 2: LED toggle output

# 5.2 LED Toggle using Open Drain configuration and enabling Internal Pull Up on STM32 Board.

Figure 3: open drain internal pull up code

Figure 4: open drain internal pull up output

# 5.3 LED Toggle using open drain configuration and external pull up on STM32 Board.

Figure 5: open drain external pull up code

Figure 6: open drain external pull up output

# 5.4 Toggle External LED using push pull configuration on STM32 Board.

Figure 7: External LED Push pull code

Figure 8: External LED Push Pull output

# 5.4 Enable internal button and toggle the LED using STM32 Board.

Figure 9: internal button led toggle code

Figure 10: internal button led toggle code

Figure 11: internal button led toggle output

# 5.6 Use external switch to toggle the LED on STM32 Board.

Figure 12: external switch led toggle

Figure 13: external switch led toggle code

Figure 14: external switch led toggle output

# 5.7 HAL INTERRUPT

Figure 15: HAL interrupt

Figure 16:  HAL interrupt

Figure 17: HAL interrupt

Figure 18:  HAL interrupt

Figure 19: HAL interrupt

Figure 20: HAL interrupt

# 5.8 HAL ADC

Figure 21: HAL ADC

Figure 22: HAL ADC

Figure 23: HAL ADC

# 5.9 HAL SPI

Figure 24: HAL SPI

Figure 25: Output of SPI

# 5.10 HAL UART

Figure 26: Output of UART